SIMATS ENGINEERING

CO5-ASSESSMENT TOOL 3

COURSE NAME:	DIGITAL  CIRCUITS	COURSE CODE: ECA02

COURSE OUTCOME COVERED

CO:5 Analyze the operation and characteristics of NMOS and PMOS transistors, CMOS inverters, and MOS device parameters used in digital circuit design (BTL 3 & 4)

Assessment Tool number                 : AT3

Assessment Tool Weightage	            : 40%

Assessment Tool Name		: HDL Code Error Debugging

Duration of this assessment		: 90 Minutes

Marks					: 20

Type of Assessment conduction       : Inside Class

Description:

- Each student will be assigned one HDL Code Error Debugging     problem 20 marks inside class. Analyze and debug one faulty analog circuit.

- Simulate the corrected circuit using VIVADO and verify results.

- Document the debugging process, observations, and conclusions.

- Upload VIVADO files, simulation results, and report to GitHub.

- Evaluation is based on fault identification, debugging approach, simulation accuracy, documentation, and GitHub submission.

| Item | Student Entry |
| --- | --- |
| 1. Given Code | module down_counter( input clk, output reg [2:0] Q); always @(negedge clk) begin Q <= Q + 1; end endmodule |
| 2. Expected code output | 7 → 6 → 5 → 4 → 3 → 2 → 1 → 0 → 7 → ... |
| 3. Observed Fault / Symptom | 0 → 1 → 2 → 3 → 4 → 5 → 6 → 7 → ... |
| 6. Error Identified | Q <= Q + 1; |
| 7. Correction Made | Q <= Q - 1; |
| 8. Corrected HDL code | module down_counter( input clk, output reg [2:0] Q ); always @(negedge clk) begin Q <= Q - 1; end endmodule |
| 9. Verification / Simulation Output |  |
| 10. Final Result & Technical Justification | The logical error was successfully identified and corrected. The original code performed up-counting because it used Q + 1. After replacing it with Q - 1, the circuit performs the required 3-bit down-counting operation on every negative edge of the clock. |